SWE4303 Computing Infrastructure

Week 3 | Session 2 Lab Handout

Inter-Subnet Communication Lab: Configure, Test, and Evidence IP Forwarding

Cisco acknowledgement: This lab is adapted for teaching in SWE4303 using Cisco Packet Tracer and selected Cisco Networking Academy lab approaches. Cisco is acknowledged as the original source of the underlying platform and lab style used here for educational delivery.

Assessment preparation note: This lab uses a fixed teaching network so that everyone can practice the same skills. Assessment 1 uses a personalised base network derived from the student ID, so do not copy these lab addresses into your assessment unless instructed.

Purpose of this lab

This lab helps you move from Week 3 lecture ideas into practical IP forwarding observation. You will build a small segmented network in Cisco Packet Tracer, configure two subnets, assign default gateways, test inter-subnet communication, and explain what the router is doing.

The purpose is not to become a routing specialist. The purpose is to understand how subnet boundaries, IP addressing, default gateways, ARP, and router forwarding make communication possible between separated network areas.

What you should be able to do by the end

- build a simple two-subnet topology in Packet Tracer

- divide a /24 network into usable subnet areas

- configure IPv4 addresses, subnet masks, and default gateways

- configure router interfaces as gateways for two LANs

- verify same-subnet and inter-subnet communication

- use simple evidence to explain why communication succeeds or fails

- diagnose and correct a basic default gateway problem

Software and resources

- Cisco Packet Tracer installed and working

- one PC or laptop with enough screen space to view the workspace and device windows comfortably

- this handout open beside Packet Tracer if possible

- time to save work carefully after each major stage

Before you start

- Open Packet Tracer and make sure you are in Realtime mode.

- Save your file early with a clear name such as Week3_Session2_YourName.pkt.

Save again after each major part. Do not use a pre-built activity file. You are expected to build the topology yourself.

Cable guidance for this lab

| Connection | Cable to use | Notes |
| --- | --- | --- |
| PC to switch | Copper Straight-Through | Use this for all PC-to-switch links. |
| Switch to router | Copper Straight-Through | Use this for switch-to-router links. |
| PC to router console | Light blue Console cable | Optional only if your tutor asks you to use console access. |

Part 0 — Understand the design brief

A small training centre has two internal network areas:

- Administration LAN

- Teaching LAN

The organisation wants these two areas separated into different subnets, but staff must still be able to communicate across the network when required.

You have been given the base network:

192.168.30.0/24

For this lab, divide the network into four equal /26 subnets. You will use the first two subnets now and leave the other two for future growth.

Subnet planning table

The table below uses fixed-length subnetting. The original 192.168.30.0/24 network has been divided into four equal /26 subnet blocks. Each /26 block contains 64 addresses. The first address in each block is the network address, the last address is the broadcast address, and the usable host range sits between them. In this lab, the first two subnet blocks are used now and the remaining two are kept for future growth.

| Subnet purpose | Network address | Prefix | Subnet mask | First usable host | Last usable host | Broadcast address |
| --- | --- | --- | --- | --- | --- | --- |
| Administration LAN | 192.168.30.0 | /26 | 255.255.255.192 | 192.168.30.1 | 192.168.30.62 | 192.168.30.63 |
| Teaching LAN | 192.168.30.64 | /26 | 255.255.255.192 | 192.168.30.65 | 192.168.30.126 | 192.168.30.127 |
| Future LAN 1 | 192.168.30.128 | /26 | 255.255.255.192 | 192.168.30.129 | 192.168.30.190 | 192.168.30.191 |
| Future LAN 2 | 192.168.30.192 | /26 | 255.255.255.192 | 192.168.30.193 | 192.168.30.254 | 192.168.30.255 |

Check your understanding

Why is 192.168.30.63 not available for a PC?

Answer: 192.168.30.63 is the broadcast address of the 192.168.30.0/26 subnet. It is reserved for sending to all hosts in the subnet, it cannot be assigned to a PC.

Why is 192.168.30.1 a sensible address for the Administration gateway?

Answer: It is the first usable host address in the Administration subnet. Using the first address for the router interface is common convention so that staff always know where the gateway is.

Why is it useful to leave two subnets unused?

Answer: It leaves room for future growth.

Part 1 — Build the topology

Devices to place

Place the following devices in Packet Tracer:

- 1 router — name it R1 (use 2911 router)

- 2 switches — name them S1 and S2 (use 2960 switch)

- 4 PCs — name them Admin-PC1, Admin-PC2, Teach-PC1, and Teach-PC2

Use a router with at least two Gigabit Ethernet interfaces. A 2911 router is suitable in many Packet Tracer versions.

If your router uses different interface names, adjust the commands carefully. For example, some routers use G0/0 and G0/1; others use G0/0/0 and G0/0/1.

Suggested layout

- Administration devices on the left

- Teaching devices on the right

- Router in the centre

- Switches between the router and PCs

Figure 1. Two-subnet topology to build in Packet Tracer

Connect the devices

Use Copper Straight-Through cables.

| Link | Suggested interface |
| --- | --- |
| Admin-PC1 to S1 | PC FastEthernet0 to S1 FastEthernet0/6 |
| Admin-PC2 to S1 | PC FastEthernet0 to S1 FastEthernet0/18 |
| Teach-PC1 to S2 | PC FastEthernet0 to S2 FastEthernet0/6 |
| Teach-PC2 to S2 | PC FastEthernet0 to S2 FastEthernet0/18 |
| S1 to R1 | S1 GigabitEthernet0/1 to R1 GigabitEthernet0/0 |
| S2 to R1 | S2 GigabitEthernet0/1 to R1 GigabitEthernet0/1 |

Save your Packet Tracer file before continuing.

Part 2 — Configure the addressing

Addressing table

| Device | Interface | IPv4 address | Subnet mask | Default gateway |
| --- | --- | --- | --- | --- |
| R1 | G0/0 | 192.168.30.1 | 255.255.255.192 | N/A |
| R1 | G0/1 | 192.168.30.65 | 255.255.255.192 | N/A |
| S1 | VLAN 1 | 192.168.30.2 | 255.255.255.192 | 192.168.30.1 |
| S2 | VLAN 1 | 192.168.30.66 | 255.255.255.192 | 192.168.30.65 |
| Admin-PC1 | NIC | 192.168.30.10 | 255.255.255.192 | 192.168.30.1 |
| Admin-PC2 | NIC | 192.168.30.11 | 255.255.255.192 | 192.168.30.1 |
| Teach-PC1 | NIC | 192.168.30.74 | 255.255.255.192 | 192.168.30.65 |
| Teach-PC2 | NIC | 192.168.30.75 | 255.255.255.192 | 192.168.30.65 |

Task 1 — Configure the PCs

For each PC:

- Click the PC.

- Open Desktop > IP Configuration.

- Enter the IPv4 address, subnet mask, and default gateway from the addressing table.

Check carefully that Administration PCs use gateway 192.168.30.1 and Teaching PCs use gateway 192.168.30.65.

Task 2 — Configure switch management addresses

The switches operate here as local switching devices. They forward frames inside each LAN; they do not route traffic between subnets. However, giving them management IP addresses lets you test whether they are reachable and reinforces gateway configuration.

Task 2.1 — Configure S1

S1 belongs to the Administration LAN. Its management address must therefore sit inside the Administration subnet, and its default gateway must be the Administration router interface.

Switch> enable Switch# configure terminal Switch(config)# hostname S1 S1(config)# no ip domain-lookup S1(config)# interface vlan 1 S1(config-if)# ip address 192.168.30.2 255.255.255.192 S1(config-if)# no shutdown S1(config-if)# exit S1(config)# ip default-gateway 192.168.30.1 S1(config)# end S1# copy running-config startup-config

What the S1 configuration is doing

| Command or command group | Why it is used |
| --- | --- |
| hostname S1 | Gives the switch a clear device name. |
| interface vlan 1 | Selects the switch management interface used in this simple lab. |
| ip address 192.168.30.2 255.255.255.192 | Gives S1 a management address in the Administration subnet. |
| no shutdown | Activates the management interface. |
| ip default-gateway 192.168.30.1 | Tells S1 where to send management traffic if the destination is outside its local subnet. |
| copy running-config startup-config | Saves the switch configuration. |

Task 2.2 — Configure S2

S2 belongs to the Teaching LAN. Its management address must sit inside the Teaching subnet, and its default gateway must be the Teaching router interface.

Switch> enable Switch# configure terminal Switch(config)# hostname S2 S2(config)# no ip domain-lookup S2(config)# interface vlan 1 S2(config-if)# ip address 192.168.30.66 255.255.255.192 S2(config-if)# no shutdown S2(config-if)# exit S2(config)# ip default-gateway 192.168.30.65 S2(config)# end S2# copy running-config startup-config

What the S2 configuration is doing

| Command or command group | Why it is used |
| --- | --- |
| interface vlan 1 | Selects the switch management interface. |
| ip address 192.168.30.66 255.255.255.192 | Gives S2 a management address in the Teaching subnet. |
| ip default-gateway 192.168.30.65 | Points S2 to the Teaching LAN gateway for non-local management traffic. |
| copy running-config startup-config | Saves the switch configuration. |

Check your understanding

Why is S1 using 192.168.30.1 as its default gateway, while S2 is using 192.168.30.65?

Answer: Each switch must use the router interface that sits inside its own subnet. S1 is in the Administration subnet, so its gateway is R1 G0/0 (192.168.30.1); S2 is in the Teaching subnet, so its gateway is R1 G0/1 (192.168.30.65).

Why do switches have management IP addresses even though they are not routing between subnets in this lab?

Answer: The management address lets you reach the switch over the network (ping, remote management) and test reachability. The switch still forwards frames without an IP - the address is only for managing the device itself.

Task 3 — Configure router R1

In this step, you are configuring R1 so that it can act as the default gateway for both subnet areas.

One router interface will belong to the Administration LAN, and the other router interface will belong to the Teaching LAN.

Click R1 > CLI. Enter the following commands. Adjust interface names only if your router uses different names.

Router> enable Router# configure terminal Router(config)# hostname R1 R1(config)# no ip domain-lookup R1(config)# interface gigabitethernet0/0 R1(config-if)# description Gateway for Administration LAN R1(config-if)# ip address 192.168.30.1 255.255.255.192 R1(config-if)# no shutdown R1(config-if)# exit R1(config)# interface gigabitethernet0/1 R1(config-if)# description Gateway for Teaching LAN R1(config-if)# ip address 192.168.30.65 255.255.255.192 R1(config-if)# no shutdown R1(config-if)# end R1# copy running-config startup-config

If Packet Tracer asks for the destination filename, press Enter.

What this configuration is doing

| Command or command group | Why it is used |
| --- | --- |
| enable | Moves from basic user mode into privileged mode so that configuration commands can be used. |
| configure terminal | Enters global configuration mode, where device settings can be changed. |
| hostname R1 | Gives the router a clear name so outputs and prompts are easier to read. |
| no ip domain-lookup | Prevents Packet Tracer from pausing while trying to treat mistyped commands as domain names. |
| interface gigabitethernet0/0 | Selects the router interface connected to the Administration LAN. |
| description Gateway for Administration LAN | Documents what the interface is used for. This helps later troubleshooting. |
| ip address 192.168.30.1 255.255.255.192 | Assigns the gateway address for the Administration subnet. |
| no shutdown | Turns the router interface on. Without this, the interface may remain administratively down. |
| interface gigabitethernet0/1 | Selects the router interface connected to the Teaching LAN. |
| ip address 192.168.30.65 255.255.255.192 | Assigns the gateway address for the Teaching subnet. |
| copy running-config startup-config | Saves the configuration so it is not lost if the device restarts. |

Check your understanding

Which command actually gives the Administration LAN its gateway address?

Answer: ip address 192.168.30.1 255.255.255.192, entered on interface gigabitethernet0/0. That command puts the gateway address on the interface that faces the Administration LAN.

Why do the two router interfaces need addresses from different subnets?

Answer: Each interface is the default gateway of a different LAN, so each must hold an address inside that LAN's subnet.

What would happen if no shutdown was not entered on an interface?

Answer: The interface would stay administratively down, no traffic would pass through it, and the connected subnet would be unreachable

Save your Packet Tracer file.

Part 3 — Verify local and inter-subnet communication

Test communication in stages. Do not jump straight to cross-subnet testing. First prove that each local area works.

Why the tests are staged

The tests below move from simple to more complex. This is a troubleshooting habit, not just a worksheet sequence.

| Stage | What you are checking | Why it matters |
| --- | --- | --- |
| Same-subnet ping | Can two devices communicate inside the same LAN? | Confirms local addressing and switching are working. |
| Ping the gateway | Can a device reach its local router interface? | Confirms the host has a working path out of its subnet. |
| Inter-subnet ping | Can traffic move between the two subnet areas? | Confirms the router interfaces and default gateway logic are working. |

Test 1 — Same-subnet communication

On Admin-PC1, open Desktop > Command Prompt and run:

ping 192.168.30.11

On Teach-PC1, run:

ping 192.168.30.75

| Test | Successful? | Evidence or note |
| --- | --- | --- |
| Admin-PC1 to Admin-PC2 | Yes | ping 192.168.30.11: 4/4 replies, 0% loss, TTL=128 |
| Teach-PC1 to Teach-PC2 | Yes | ping 192.168.30.75: 4/4 replies, 0% loss, TTL=128 |

Test 2 — Ping the local default gateway

On Admin-PC1, run:

ping 192.168.30.1

On Teach-PC1, run:

ping 192.168.30.65

| Test | Successful? | What does this prove? |
| --- | --- | --- |
| Admin-PC1 to 192.168.30.1 | Yes | Admin-PC1 can reach its local router interface - it has a working path out of its subnet. |
| Teach-PC1 to 192.168.30.65 | Yes | Teach-PC1 can reach its local router interface - gateway reachable, path out of the subnet works. |

Check your understanding

Why should a PC be able to ping its own default gateway before you test communication with another subnet?

Answer: Pinging the gateway proves the host has a working path out of its own subnet

Test 3 — Inter-subnet communication

On Admin-PC1, run:

ping 192.168.30.74

On Teach-PC1, run:

ping 192.168.30.10

| Test | Successful? | What does this prove? |
| --- | --- | --- |
| Admin-PC1 to Teach-PC1 | Yes | First request timed out (ARP), then 3 replies, TTL=127 - R1 forwards between the subnets. |
| Teach-PC1 to Admin-PC1 | Yes | 4/4 replies, 0% loss, TTL=127 - routing works in both directions. |

If the first ping fails but later pings work, do not panic. The first attempt may be affected while devices learn required addressing information.

Part 4 — Verify the router’s forwarding role

Click R1 > CLI. Enter:

R1# show ip interface brief

What this command is checking

| Command | Why it is used |
| --- | --- |
| show ip interface brief | Gives a short summary of router interfaces, assigned IP addresses, and whether each interface is up. |

Record what you see.

| Interface | IP address | Status | Protocol |
| --- | --- | --- | --- |
| G0/0 | 192.168.30.1 | up | up |
| G0/1 | 192.168.30.65 | up | up |

Why must both router interfaces be up before inter-subnet communication can work?

Answer: The router only forwards between interfaces that are up. If either interface is down, one subnet is disconnected from the router, so packets have nowhere to go and inter-subnet communication fails.

Now enter:

R1# show ip route

Look for connected routes beginning with C.

What this command is checking

| Command | Why it is used |
| --- | --- |
| show ip route | Shows the networks the router currently knows how to reach. In this lab, the important routes are directly connected networks. |
| Connected route (C) | Shows a network that is directly attached to one of the router's active interfaces. |

Which two connected networks does R1 know about?

Answer: 192.168.30.0/26  and 192.168.30.64/26 shown as connected routes.

Why does R1 not need a dynamic routing protocol in this lab?

Answer: Both subnets are directly connected to R1, so it automatically knows a route to each. Dynamic routing protocols are only needed to learn about networks that are not directly attached.

Part 5 — Observe ARP and gateway behaviour

In this part, you will observe what a PC learns when it communicates with a device in another subnet.

ARP is included here because IP addressing alone does not place a frame onto the local link. Even when a device is sending traffic to a remote subnet, it still needs a local MAC address for the next hop.

On Admin-PC1, open Command Prompt. Clear the ARP table:

arp -d

Now ping a remote device:

ping 192.168.30.74

Then view the ARP table:

arp -a

What these commands are doing

| Command | Why it is used |
| --- | --- |
| arp -d | Clears learned ARP entries so you can observe what is learned again. |
| ping 192.168.30.74 | Generates traffic from the Administration subnet to the Teaching subnet. |
| arp -a | Displays the ARP entries known by the PC after communication has been attempted. |

Record one relevant entry.

| IP address shown in ARP table | MAC address shown | What device do you think this belongs to? |
| --- | --- | --- |
| 192.168.30.1 | 000A.4136.5701 (dynamic) | R1's GigabitEthernet0/0 - Admin-PC1's local default gateway. |

When Admin-PC1 sends traffic to Teach-PC1, does it need the MAC address of Teach-PC1 directly, or the MAC address of its local gateway?

Answer: It needs the MAC address of its local gateway

Why is this important for understanding communication across subnets?

Answer: It shows that IP delivers end-to-end but frames are delivered hop-by-hop. A host only ever needs the MAC of the next hop on its own link; the router re-frames the packet for the next LAN.

Optional router ARP check

On R1, enter:

R1# show arp

What this command is checking

| Command | Why it is used |
| --- | --- |
| show arp | Shows the IP-to-MAC mappings known by the router. This helps you see how IP forwarding still depends on local frame delivery at each link. |

What does the router’s ARP table help show?

Answer: It shows the IP-to-MAC mappings R1 has learned on each connected LAN (e.g. Admin-PC1 and Teach-PC1).

Part 6 — Troubleshoot a default gateway problem

In this part you will create and then fix a simple fault.

This is deliberate. Good infrastructure work is not only about making a configuration work once. It is also about recognising why a configuration fails and proving that the fix is correct.

On Teach-PC2, change the default gateway from 192.168.30.65 to 192.168.30.1. Do not change the IP address or subnet mask.

Why this fault matters

Teach-PC2 still belongs to the Teaching subnet. Its default gateway should therefore be the router interface inside the Teaching subnet. 192.168.30.1 is the Administration gateway, so it is not the correct local gateway for Teach-PC2.

Test after the fault

From Teach-PC2, try:

ping 192.168.30.74

Then try:

ping 192.168.30.10

| Test | Successful? | What does the result suggest? |
| --- | --- | --- |
| Teach-PC2 to Teach-PC1 | Yes - 4/4 replies (local delivery does not use the gateway). | Local, same-subnet traffic still works. |
| Teach-PC2 to Admin-PC1 | No - Request timed out x4, 100% loss. | Remote traffic fails because it is sent towards a gateway that is not valid in this subnet. |

Why can a wrong default gateway allow local communication but break remote communication?

Answer: Local traffic never uses the gateway - the host ANDs the destination with its mask, sees it is on-link, and delivers directly. Remote traffic must be sent to the gateway; if that address is wrong (or unreachable from this subnet), packets to other subnets are never delivered.

Fix the problem

Change the default gateway on Teach-PC2 back to 192.168.30.65.

Repeat the remote ping:

ping 192.168.30.10

Result and brief explanation

Answer: After changing the gateway back to 192.168.30.65, ping 192.168.30.10 succeeded: 4/4 replies, 0% loss, TTL=127. Brief explanation: Teach-PC2 again sends remote traffic to the correct local gateway (R1 G0/1), which forwards it to the Administration subnet and back.

Brief explanation: ________________________________________________________________

Troubleshooting record

Complete this short record. This is the type of evidence logic that will help later in Assessment 1.

| Item | Your note |
| --- | --- |
| Fault introduced | Default gateway on Teach-PC2 changed from 192.168.30.65 to 192.168.30.1 (Administration gateway). |
| Symptom observed | Teach-PC2 could still ping Teach-PC1 (192.168.30.74) but pings to Admin-PC1 (192.168.30.10) timed out - 100% loss. |
| Cause identified | Wrong default gateway: 192.168.30.1 is not inside the Teaching subnet, so Teach-PC2 had no usable next hop for remote traffic. |
| Fix applied | Default gateway set back to 192.168.30.65 in Desktop > IP Configuration. |
| Evidence that the fix worked | Repeat ping 192.168.30.10 succeeded: 4/4 replies, 0% loss, TTL=127. |

Part 7 — From configuration to explanation

Use your evidence to answer the following questions.

Same subnet or different subnet: How does a host decide whether a destination is local or remote?

Answer: Same network = local (deliver directly via ARP); different network = remote (send to the default gateway).

Default gateway: What is the role of the default gateway?

Answer: It is the router interface a host sends all traffic to when the destination is outside the host's own subnet - the exit point from the local network.

Router forwarding: What does the router do that the switches cannot do in this lab?

Answer: R1 forwards packets between the two subnets (routing between broadcast domains). The switches only deliver frames inside one LAN;

ARP: Why is ARP still involved even when IP addressing is being used?

Answer: IP identifies the end devices, but every frame must still be delivered on a physical link using MAC addresses. ARP maps the next hop's IP (a local host or the gateway) to its MAC so the frame can be built.

Segmentation: Why might an organisation separate Administration and Teaching devices into different subnets?

Answer: Separation limits broadcast traffic, groups devices by role, makes addressing and troubleshooting clearer, and lets policies/security be applied per area while the router still connects them when needed.

Optional challenge — Use Simulation Mode

Complete this only if you have finished the main lab or your tutor asks you to do it.

- Clear the ARP table on Admin-PC1.

- Switch Packet Tracer to Simulation mode.

- From Admin-PC1, ping Teach-PC1.

- Use Capture / Forward to step through the communication.

- Click the PDU envelopes and inspect the frame/MAC information and the IP packet information.

Look for:

- whether the source and destination IP addresses remain linked to the original sender and receiver

- whether the MAC addresses change as traffic passes through the router

- where the router becomes involved

What does Simulation Mode help you see that normal ping output does not?

Answer: It shows the frame headers hop by hop: the source and destination IP addresses stay the same end-to-end, but the MAC addresses are rewritten at the router (PC-to-R1 MACs, then R1-to-destination MACs). A ping only reports success and timing - it hides where the router steps in.

Evidence checklist

Before you finish, make sure you have the following evidence.

| Evidence item | Completed |
| --- | --- |
| Screenshot of the completed topology | [ X ] |
| Completed subnet planning table | [ X ] |
| PC IP configuration evidence for one Administration PC | [ X ] |
| PC IP configuration evidence for one Teaching PC | [ X ] |
| Router interface configuration or show ip interface brief output | [ X ] |
| show ip route output showing connected networks | [ X ] |
| Successful same-subnet ping evidence | [ X ] |
| Successful inter-subnet ping evidence | [ X ] |
| ARP table evidence from a PC | [ X ] |
| Default gateway troubleshooting evidence | [ X ] |
| Short written answers for the reasoning questions | [ X ] |

If you get stuck

Check in this order:

- Are the cables connected to the correct devices?

- Are the router interfaces turned on with no shutdown?

- Is each PC in the correct subnet?

- Is each subnet mask correct?

- Is each default gateway the router interface in the same subnet as the PC?

- Can the PC ping its own gateway?

- Does show ip interface brief show both router interfaces as up?

- Does show ip route show both connected networks?

Do not change many settings at the same time. Change one thing, test it, and then continue.

Key idea from this lab

A segmented network needs more than switches. Hosts use IP address and subnet mask logic to decide whether a destination is local or remote. When the destination is remote, the host sends traffic to its default gateway. The router then forwards traffic between the connected subnets.

Consolidate and look ahead

After the lab, complete the Week 3 quiz on IPv4 addressing, subnet masks, subnetting, default gateways, ARP, and forwarding decisions.

Self-study: produce a small addressing table for a given scenario and explain why the gateway addresses were selected.

Keep your Packet Tracer file because it may help you when preparing for the Assessment 1 Packet Tracer proof of concept.

Appendix

Screenshots: